Do we need so much fuel for Space Travel?

Resources From Microsoft Copilot

Check back from previous conversation, I want to launch a space craft to the land on the moon, the fuel needed is for the rocket to launch the space craft to the space like the Columbia and Atlantis, then in the space I just ignite the engine once and due to vacuum state (frictionless), the space craft will go straight to the moon's orbit. The fuel needed for the space craft is just for moon landing and launching back to earth, that means fuel needed for my space craft is not too much. please comment?

let’s break it down with the mechanics of orbital flight — because the “ignite once and coast straight to the Moon” concept misses a few critical realities.

Launch vs. Deep-Space Travel

- Launch fuel: As we discussed before, ~90% of the rocket’s fuel is burned just to reach Low Earth Orbit (LEO). That part is unavoidable — you need huge thrust to fight gravity and atmosphere.

- Trans-Lunar Injection: Once in orbit, you do a burn to set your spacecraft on a curved path toward the Moon. It’s not a straight line — it’s an orbital transfer. This burn is relatively small compared to launch.

Why “ignite once” doesn’t work?

- Orbital mechanics: You can’t just point at the Moon and fire. The Moon is moving ~1 km/s around Earth, so you must aim ahead of it. That requires precise timing and trajectory shaping.

- Lunar capture: If you don’t slow down when you arrive, you’ll fly past the Moon. A second burn is needed to enter lunar orbit.

- Landing: To touch down, you must cancel all orbital velocity — another major burn.

- Return: Leaving the Moon and coming back to Earth requires yet another burn.

So while coasting is free in vacuum, each change of orbit requires fuel. That’s why Apollo missions had multiple burns.

Fuel Distribution (Apollo-style missions)

| Phase | Approx. Fuel Use | Purpose |
| --- | --- | --- |
| Launch to LEO | ~85–90% | Escape Earth’s gravity |
| TLI burn | ~3% | Set course to Moon |
| Lunar orbit insertion | ~2% | Capture by Moon |
| Lunar landing | ~2% | Descent |
| Lunar ascent | ~1% | Return to orbit |
| Earth return | <1% | Small corrections |

So yes — the spacecraft itself doesn’t need “too much” fuel compared to the launch rocket. But it still needs enough for at least 4 separate burns.

Comment on your concept

Your plan of “big rocket to orbit + small shuttle to Moon” is partially correct:

- The shuttle wouldn’t need a huge fuel tank like the launch rocket.

- But it cannot rely on a single ignition. It must carry enough propellant for injection, capture, landing, ascent, and return.

- That’s why Apollo used a Service Module (for orbital burns) and a Lunar Module (for landing and ascent). Each had its own fuel.